Standard Operating Procedure: Lighting Installation, Use and Maintenance

1.	Purpose

To establish standardized procedures for the installation, operation, and maintenance of horticultural lighting systems in compliance with NYS OCM energy efficiency requirements and safety standards.

2.	Scope

This SOP applies to all personnel involved in the installation, operation, or maintenance of cultivation lighting equipment in licensed cannabis facilities.

3.	Prerequisites

Required prerequisites:
• Understanding of NYS OCM lighting efficiency standards (Section 125.1(b))
• Electrical systems must meet local building codes
• Personnel must be trained on electrical safety
• Manufacturer specifications must be reviewed

4.	Responsibilities

Key personnel:
• Facility Manager: Ensures compliance with OCM lighting standards
• Cultivation Manager: Specifies lighting requirements for cultivation areas
• Maintenance Technician: Performs installation and maintenance
• Licensed Electrician: Required for electrical work

5.	Materials

Required materials:
• OCM-compliant lighting fixtures meeting PPE standards:
  - Indoor: Minimum 2.2 μmol/J photosynthetic photon efficacy
  - Mixed-Light: Minimum 1.7 μmol/J photosynthetic photon efficacy
• Light meters (PAR/PPFD capable)
• Light hangers and adjustment hardware
• Timers or automated lighting controllers
• Safety equipment (glasses, gloves)
• Maintenance logs

6.	Procedure

6.1 Lighting Selection and Compliance
   a. Verify all fixtures meet NYS OCM PPE standards:
      - Indoor cultivation: ≥2.2 μmol/J
      - Mixed-light cultivation: ≥1.7 μmol/J
   b. Obtain manufacturer specifications and certifications
   c. Document compliance for audit requirements
   d. Select LED fixtures to reduce energy consumption
   e. Calculate total wattage requirements per room

6.2 Pre-Installation Requirements
   a. Consult licensed electrician for electrical assessment
   b. Verify circuit capacity for planned load
   c. Submit utility service request per Section 125.2(c)(1)(i)
   d. Ensure compliance with local fire codes
   e. Plan lighting layout for uniform coverage

6.3 Installation Procedures
   a. Turn off all power before installation
   b. Install mounting hardware according to specifications
   c. Hang fixtures at manufacturer-recommended heights
   d. Secure all electrical connections properly
   e. Install timers or connect to automation system
   f. Test each fixture individually before full operation
   g. Document installation date and fixture details

6.4 Operating Parameters by Growth Stage
   a. Seedlings/Clones:
      - PPFD: 100-300 μmol/m²/s
      - Height: Higher positioning (reduced intensity)
      - Duration: 18-24 hours
   b. Vegetative:
      - PPFD: 400-600 μmol/m²/s
      - Duration: 18-24 hours
   c. Flowering:
      - PPFD: 600-900 μmol/m²/s
      - Duration: 12 hours on/12 hours off (strict)

6.5 Light Measurement and Mapping
   a. Use PAR meter to measure PPFD at canopy level
   b. Create light map for each room
   c. Identify and correct hot spots or dead zones
   d. Adjust fixture heights for uniform coverage
   e. Document readings and adjustments

6.6 Safety Protocols
   a. Never look directly at high-intensity lights
   b. Wear eye protection when working around lights
   c. Allow fixtures to cool before handling
   d. Maintain clearance from plants and flammable materials
   e. Use ground fault protection on circuits

6.7 Routine Maintenance
   a. Weekly: Visual inspection of fixtures and connections
   b. Monthly: Clean lenses/reflectors of dust and debris
   c. Quarterly: Verify light output with PAR meter
   d. Annually: Full system inspection and performance audit
   e. Replace fixtures when output drops >20%

6.8 Documentation and Reporting
   a. Maintain equipment inventory with specifications
   b. Record installation dates and locations
   c. Log maintenance activities and repairs
   d. Track energy consumption per room
   e. Retain records for OCM audit compliance

7.	References

• NYS OCM Regulations Section 125.1(b) - Horticultural Lighting Standards
• Energy and Environmental Plan Guidance (OCM, November 2024)
• Section 125.2(a)(6)(vii) - Description of Lighting Equipment
• Local electrical and fire codes

8.	Definitions

• PPE (Photosynthetic Photon Efficacy): Efficiency measure (μmol/J)
• PPFD: Photosynthetic Photon Flux Density (μmol/m²/s)
• PAR: Photosynthetically Active Radiation (400-700nm)
• Mixed-Light: Combination of natural sunlight and artificial lighting

9.	Sources

Information obtained from:
• NYS Office of Cannabis Management: https://cannabis.ny.gov/
• Energy and Environmental Plan Guidance (OCM, November 2024)
• Medical Cannabis Home Cultivation Guide (OCM)

Related SOPs and Cross-References

Category: ENVIRONMENT - NYS OCM

The following SOPs are related to this procedure:

GEN-012: Preventive Maintenance - Equipment maintenance

GEN-050: OSHA Electrical Safety - Electrical safety

CULT-005: Vegetative Phase - Light schedules

Note: This SOP is part of a comprehensive cannabis compliance system. SOPs marked "(OTHER STATES)" contain recommendations from other state programs and are suggestions, not NYS OCM requirements.