Standard Operating Procedure: Sample Retention

1.	Purpose

This Standard Operating Procedure establishes guidelines for the retention, storage, and management of reference and retain samples from cannabis production in compliance with NYS Office of Cannabis Management (OCM) requirements.

2.	Scope

This SOP applies to all retain samples collected from production batches at licensed cannabis facilities in New York State, including flower, extracts, edibles, and other finished products.

3.	Prerequisites

• Understanding of sample retention requirements
• Knowledge of proper storage conditions
• Training on sample handling
• Access to sample storage areas
• Familiarity with batch documentation

4.	Responsibilities

Quality Assurance Manager:
• Establish retention requirements
• Approve sample requests
• Oversee retention program
• Determine sample disposition

Quality Control:
• Collect retain samples
• Label and store samples
• Maintain sample inventory
• Retrieve samples when requested

Laboratory:
• Analyze retain samples when required
• Report sample test results
• Support investigations

Production:
• Support sample collection
• Provide batch information
• Notify QC of batch completion

5.	Materials

• Sample containers (appropriate for product type)
• Labels (product and batch information)
• Sample storage areas (controlled temperature)
• Sample log/inventory database
• Sample request forms
• Tamper-evident seals
• Chain of custody forms

6.	Procedure

6.1 Sample Retention Requirements
• Retain samples from each production batch
• Sample quantity requirements:
  - Sufficient for at least 2 complete analyses
  - Minimum per product type as specified
  - Account for stability testing if required
• Retain samples for:
  - Minimum 1 year beyond product expiration
  - Or 3 years from batch production, whichever is longer
  - Per specific OCM requirements

6.2 Sample Collection
• Collect samples at appropriate point:
  - After final packaging (preferred)
  - Representative of batch
• Sample collector must:
  - Follow aseptic technique where applicable
  - Use appropriate containers
  - Collect correct quantity
  - Avoid contamination
• Document collection:
  - Date and time
  - Batch/lot number
  - Collector name
  - Quantity collected

6.3 Sample Labeling
• Label each sample with:
  - Product name
  - Batch/lot number
  - Date of manufacture
  - Expiration date (if applicable)
  - Sample collection date
  - Storage conditions
  - Retain until date
• Use durable, legible labels
• Apply tamper-evident seals

6.4 Sample Storage
• Store samples under appropriate conditions:
  - Same conditions as commercial product
  - Controlled temperature as required
  - Protected from light if light-sensitive
  - Secure from unauthorized access
• Organize storage area:
  - By product type
  - By retention date
  - Clearly identified locations
• Monitor storage conditions
• Document environmental excursions

6.5 Sample Inventory Management
• Maintain inventory of all retain samples:
  - Product identification
  - Batch/lot number
  - Collection date
  - Storage location
  - Retain until date
  - Current status
• Update inventory when samples:
  - Added to storage
  - Removed for testing
  - Disposed of at expiration
• Review inventory quarterly

6.6 Sample Retrieval
• Retrieve samples only with authorization
• Request must include:
  - Reason for retrieval
  - Quantity needed
  - Requestor information
  - Authorization approval
• Document retrieval:
  - Date
  - Sample identification
  - Quantity removed
  - Purpose
  - Person retrieving
• Maintain chain of custody

6.7 Sample Testing
• Test retain samples when:
  - Investigating complaints
  - Investigating stability
  - Regulatory request
  - Supporting recall decisions
• Compare results to original testing
• Document all testing results
• Link results to batch records

6.8 Sample Disposition
• Dispose of samples after retention period
• Disposition process:
  - Verify retention period complete
  - Confirm no pending investigations
  - Document samples to dispose
  - Dispose per waste disposal SOP
  - Update inventory
• Do not dispose if:
  - Active investigation involving batch
  - Pending regulatory inquiry
  - Active complaint investigation

6.9 Documentation
• Maintain retain sample records:
  - Collection records
  - Storage location log
  - Retrieval records
  - Testing results
  - Disposition records
• Retain records per document control SOP
• Link to batch production records
• Make available for regulatory inspection

7.	References

• NYS OCM GMP Guidance
• NYS OCM Record Retention Requirements
• USP General Chapters on Stability
• Source: https://cannabis.ny.gov/

8.	Definitions

Retain Sample: A sample from a production batch kept for future reference, testing, or investigation.

Reference Sample: A sample used to verify identity and characteristics of materials.

Chain of Custody: Documentation that tracks the possession and handling of samples.

Batch/Lot: A specific quantity of product produced during a defined production cycle.

Retention Period: The required length of time samples must be kept before disposal.

Related SOPs and Cross-References

Category: QUALITY - NYS OCM

The following SOPs are related to this procedure:

PROC-015: Laboratory Testing - Testing samples

GEN-036: Storage Inventory - Sample storage

PROC-029: Quality Control - QC samples

Note: This SOP is part of a comprehensive cannabis compliance system. SOPs marked "(OTHER STATES)" contain recommendations from other state programs and are suggestions, not NYS OCM requirements.