Standard Operating Procedure: Energy and Resource Tracking

1.	Purpose

This SOP establishes procedures for tracking energy and resource consumption as required by 9 NYCRR Part 125 Energy and Environmental Plan requirements and to support sustainability reporting.

2.	Scope

This procedure applies to all NYS licensed cannabis facilities required to submit Energy and Environmental Plans and track utility consumption, water usage, and waste generation.

3.	Prerequisites

• Utility account information available
• Metering equipment installed (if required)
• Tracking spreadsheet or software system
• Understanding of Part 125 requirements
• Baseline data established

4.	Responsibilities

• Facility Manager: Oversee tracking; submit reports
• Operations Staff: Record resource usage
• Maintenance: Maintain metering equipment
• Finance: Provide utility bill information
• Compliance: Verify accuracy; submit to OCM

5.	Materials and Equipment

• Utility meters (electricity, gas, water)
• Sub-metering equipment (if applicable)
• Tracking spreadsheet or software
• Utility invoices/records
• Calculator or calculation software
• Reporting templates

6.	Procedure

1. Energy Tracking Requirements
   a. Track total electricity consumption (kWh)
   b. Track natural gas consumption (therms)
   c. Track fuel consumption (if applicable)
   d. Calculate energy use intensity (EUI)
   e. Monitor horticultural lighting efficiency (PPE)

2. Water Tracking
   a. Record total water consumption (gallons)
   b. Track water source (municipal, well)
   c. Monitor irrigation water separately
   d. Track wastewater generation
   e. Calculate water use efficiency

3. Data Collection Methods
   a. Monthly utility bill review
   b. Sub-meter readings (if equipped)
   c. Equipment runtime logging
   d. Manual readings where needed
   e. Automated monitoring systems (if available)

4. Recording Procedures
   a. Enter data into tracking system
   b. Record by billing period/month
   c. Note any unusual consumption
   d. Calculate running totals
   e. Compare to previous periods
   f. Document data sources

5. Energy Calculations
   a. Total site electricity (kWh)
   b. Lighting energy (kWh) if sub-metered
   c. HVAC energy (kWh) if sub-metered
   d. Energy per unit of production
   e. Greenhouse gas emissions (CO2e)

6. Horticultural Lighting Tracking
   a. Document installed lighting capacity
   b. Calculate Photosynthetic Photon Efficacy (PPE)
   c. Track hours of operation
   d. Calculate lighting energy use
   e. Compare to Part 125 thresholds

7. HVAC Efficiency Tracking
   a. Document equipment specifications
   b. Track refrigerant types and quantities
   c. Log any refrigerant additions
   d. Document maintenance activities
   e. Monitor for low-GWP compliance

8. Waste Tracking
   a. Track organic waste volumes
   b. Record cannabis waste quantities
   c. Track packaging waste
   d. Monitor recycling quantities
   e. Document waste disposal methods

9. Reporting
   a. Prepare monthly summary reports
   b. Calculate quarterly totals
   c. Compare to targets/benchmarks
   d. Identify improvement opportunities
   e. Submit to OCM as required

10. Data Retention
    a. Retain all utility records
    b. Keep tracking spreadsheets/data
    c. Maintain equipment logs
    d. Store for minimum 5 years
    e. Make available for audit

7.	References

• 9 NYCRR Part 125 Section 125.3 (Energy and Environmental Plan)
• NYS Climate Leadership and Community Protection Act
• EPA ENERGY STAR Portfolio Manager
• cannabis.ny.gov - Environmental Requirements
• Utility company reporting resources

8.	Definitions

• EUI: Energy Use Intensity - energy per square foot
• PPE: Photosynthetic Photon Efficacy - μmol/J lighting efficiency
• GWP: Global Warming Potential - refrigerant climate impact
• CO2e: Carbon dioxide equivalent - greenhouse gas metric
• kWh: Kilowatt-hour - unit of electricity consumption

Related SOPs and Cross-References

Category: ENVIRONMENT - NYS OCM

The following SOPs are related to this procedure:

GEN-052: Energy Environmental Plan Development - Data feeds into EEP

GEN-053: PowerScore Benchmark Reporting - Data for annual reporting

GEN-055: Greenhouse Gas Emissions Management - Data for emissions calculations

CULT-007: Lighting Installation Use Maintenance - Lighting energy tracking

CULT-008: HVAC Refrigerant Compliance - HVAC energy tracking

CULT-012: Water Efficiency Conservation - Water resource tracking

PROC-017: Cannabis Waste Disposal - Waste tracking

Note: This SOP is part of a comprehensive cannabis compliance system. SOPs marked "(OTHER STATES)" contain recommendations from other state programs and are suggestions, not NYS OCM requirements.