# Standard Operating Procedure: Energy and Environmental Plan Development

# 1.	Purpose

This SOP establishes procedures for developing, submitting, and maintaining the Energy and Environmental Plan (EEP) required by NYS Office of Cannabis Management under 9 NYCRR Part 125 for all licensed cannabis operations.

# 2.	Scope

This procedure applies to all NYS licensed cannabis facilities required to submit an Energy and Environmental Plan to the Office of Cannabis Management, including cultivators, processors, and distributors.

# 3.	Prerequisites

• Understanding of 9 NYCRR Part 125 requirements

• Access to facility energy and resource data

• Completed baseline resource tracking

• Knowledge of sustainability compliance timeline

• Administrative access to OCM licensing portal

# 4.	Responsibilities

Compliance Manager:

• Oversee EEP development and submission

• Ensure plan meets all OCM requirements

• Coordinate with departments for data collection

• Submit plan and updates to OCM

• Respond to OCM feedback or requests

Facility Manager:

• Provide facility specifications and data

• Implement plan requirements operationally

• Ensure staff adherence to environmental goals

• Report progress on sustainability targets

Operations Staff:

• Collect required resource usage data

• Follow environmental procedures in plan

• Document resource consumption accurately

• Report issues affecting environmental compliance

# 5.	Materials and Equipment

• OCM Energy and Environmental Plan Guidance document

• Facility energy consumption records (12+ months)

• Water usage records and bills

• Equipment specifications (lighting, HVAC)

• Waste generation and disposal records

• Building square footage documentation

• Canopy square footage records (cultivation)

• OCM licensing portal access credentials

# 6.	Procedure

6.1 Plan Requirements Overview

EEP required for all adult-use licensees

Submit with initial license application

Update annually or when significant changes occur

Plan must address: energy use, water conservation, waste minimization, emissions management

6.2 Energy Use Documentation

Document total facility electricity consumption (kWh)

Calculate energy use intensity (kWh/sq ft annually)

For cultivators: document lighting efficiency (PPE in µmol/J)

Minimum PPE threshold: 2.2 µmol/J for horticultural lighting

Document HVAC equipment and refrigerant types

Refrigerant GWP must be ≤750 or use natural refrigerants

Document any on-site fuel combustion

Note: Fossil fuel as primary heat source is prohibited

6.3 Water Conservation Planning

Document water sources (municipal, well, collected)

Establish baseline water consumption

Set water reduction goals

Document irrigation efficiency measures

Plan for water recycling/reclamation where feasible

Include leak detection and prevention procedures

6.4 Waste Minimization Planning

Document waste streams (organic, packaging, hazardous)

Set waste reduction targets

Describe waste diversion strategies

Composting program for organic waste

Recycling program for packaging

Document disposal methods and vendors

Include tracking and documentation procedures

6.5 Greenhouse Gas Emissions

Calculate baseline emissions (CO2 equivalent)

Document emission reduction strategies

Address refrigerant management and leak prevention

Document transportation/delivery emissions if applicable

Set emissions reduction targets

6.6 Sustainable Packaging

Include Environmental Sustainability Product Packaging Plan

Document packaging materials and recyclability

Set targets for post-consumer recycled content

Plan for packaging waste reduction

Reference PLMA (Packaging, Labeling, Marketing, Advertising) compliance

6.7 Plan Submission

Complete EEP using OCM-provided template or guidance format

Include all required documentation and supporting data

Submit through OCM licensing portal

Retain copy of submitted plan and confirmation

Note: Initial submission due with license application

6.8 Annual Updates and Reporting

Review and update EEP annually

Document progress toward stated goals

Update data with current year consumption/usage

Submit updated plan per OCM schedule

Address any OCM feedback from previous submissions

6.9 Compliance Monitoring

Track performance against EEP goals monthly

Document deviations and corrective actions

Prepare for potential OCM audits

Maintain all supporting records for 5+ years

# 7.	References

• 9 NYCRR Part 125 (Energy and Environmental Plan Requirements)

• NYS Climate Leadership and Community Protection Act (CLCPA)

• OCM Energy and Environmental Plan Guidance (November 2024)

• NYS Environmental Conservation Law Section 19-0329 (Refrigerants)

• OCM Adult-Use Cannabis Regulations

# 8.	Definitions

Energy and Environmental Plan (EEP): Required document submitted to OCM outlining facility's energy use, conservation measures, and environmental compliance strategies.

Energy Use Intensity (EUI): Total energy consumption divided by building square footage, typically expressed as kWh/sq ft/year.

Photosynthetic Photon Efficacy (PPE): Measure of horticultural lighting efficiency expressed as micromoles of photons per joule (µmol/J).

Global Warming Potential (GWP): 100-year measure of a substance's contribution to climate change relative to CO2.

CLCPA: Climate Leadership and Community Protection Act - NYS law setting statewide emissions reduction targets.

# 9.	Sources

Information obtained from:

• NYS Office of Cannabis Management: https://cannabis.ny.gov/sustainability

• OCM Energy and Environmental Plan Guidance: https://cannabis.ny.gov/system/files/documents/2024/11/energy-and-environmental-plan-guidance.pdf

• NYS Department of Environmental Conservation: https://dec.ny.gov/

Related SOPs and Cross-References

Category: ENVIRONMENT - NYS OCM

The following SOPs are related to this procedure:

GEN-039: Energy Resource Tracking - Data collection for EEP

GEN-053: PowerScore Benchmark Reporting - Cultivator reporting requirement

GEN-054: Sustainable Packaging Compliance - Packaging plan component

GEN-055: Greenhouse Gas Emissions Management - Emissions component of EEP

CULT-008: HVAC Refrigerant Compliance - Low-GWP refrigerant requirements

CULT-012: Water Efficiency Conservation - Water conservation component

PROC-017: Cannabis Waste Disposal - Waste management component

Note: This SOP is part of a comprehensive cannabis compliance system. SOPs marked "(OTHER STATES)" contain recommendations from other state programs and are suggestions, not NYS OCM requirements.