Standard Operating Procedure: Ethanol Extraction

1.	Purpose

This Standard Operating Procedure establishes guidelines for ethanol extraction of cannabis to produce concentrates and oils in compliance with NYS Office of Cannabis Management (OCM) safety and processing requirements.

2.	Scope

This SOP applies to all ethanol extraction operations at licensed cannabis processing facilities in New York State, including cold and warm extraction methods.

3.	Prerequisites

• Training on ethanol extraction techniques
• Understanding of flammable liquid safety
• Knowledge of extraction equipment operation
• Familiarity with solvent handling
• Fire safety certification

4.	Responsibilities

Extraction Manager:
• Oversee extraction operations
• Ensure safety compliance
• Approve extraction parameters
• Monitor quality outcomes

Extraction Technicians:
• Execute extraction procedures
• Operate extraction equipment
• Monitor process parameters
• Maintain safety protocols

Safety Officer:
• Verify safety systems
• Conduct safety inspections
• Respond to incidents
• Maintain safety equipment

Quality Assurance:
• Verify input material quality
• Monitor residual solvent levels
• Approve product release
• Review batch records

5.	Materials

• Food-grade ethanol (190+ proof)
• Extraction vessel
• Filtration equipment
• Rotary evaporator or distillation equipment
• Freezer (for cold extraction)
• Personal protective equipment
• Fire suppression equipment
• Ventilation system
• Explosion-proof equipment
• Solvent storage cabinets

6.	Procedure

6.1 Safety Requirements
• Extraction area requirements:
  - C1D1 or C1D2 classified space
  - Explosion-proof electrical
  - Adequate ventilation
  - Fire suppression system
  - Emergency eyewash/shower
• No ignition sources in area
• Ground all equipment
• Post safety signage
• Verify all safety systems operational

6.2 Pre-Extraction Preparation
• Review batch documentation
• Verify input material approved
• Calculate solvent requirements
• Check equipment functionality
• Verify ventilation operational
• Don appropriate PPE:
  - Safety glasses
  - Nitrile gloves
  - Lab coat
  - Closed-toe shoes

6.3 Material Preparation
• Weigh input cannabis material
• Record in tracking system
• Grind or prepare material (optional)
• Place in extraction vessel
• Document material weight

6.4 Cold Ethanol Extraction
• Pre-chill ethanol to -40°F to -60°F
• Pre-chill extraction vessel if possible
• Add cold ethanol to material:
  - Ratio: 10:1 ethanol to cannabis typical
  - Adjust based on material type
• Agitate gently for 10-20 minutes
• Shorter contact time reduces chlorophyll
• Filter to remove plant material

6.5 Warm Ethanol Extraction
• Use room temperature ethanol
• Add ethanol to material
• Allow longer contact (30-60 minutes)
• Agitate periodically
• Filter to remove plant material
• Note: More chlorophyll extraction

6.6 Filtration
• Filter extract through appropriate media:
  - Initial coarse filtration
  - Fine filtration (1-5 micron)
  - Carbon filtration (optional for color)
• Collect filtrate
• Dispose of plant material per waste SOP
• Document filtration process

6.7 Solvent Recovery
• Transfer filtrate to evaporation equipment
• Set evaporator parameters:
  - Temperature: Below 120°F recommended
  - Vacuum: As appropriate for equipment
• Recover ethanol for reuse
• Monitor continuously
• Collect recovered solvent
• Continue until desired consistency

6.8 Post-Processing
• May require additional steps:
  - Winterization (if warm extraction)
  - Decarboxylation
  - Distillation
• Document all processing steps
• Test for residual solvent
• Must meet limits (<5000 ppm ethanol)

6.9 Residual Solvent Testing
• Test all batches for residual solvent
• Maximum allowable limits per OCM
• Do not release until passing
• Reprocess if necessary
• Document test results

6.10 Yield Calculation and Documentation
• Weigh final product
• Calculate extraction yield
• Record in tracking system:
  - Input weight
  - Output weight
  - Solvent used
  - Solvent recovered
• Complete batch record
• Submit samples for testing

6.11 Solvent Management
• Track all solvent use:
  - Fresh solvent in
  - Recovered solvent
  - Waste solvent
• Store solvent properly:
  - Approved flammable storage
  - Grounded containers
  - Temperature controlled
• Dispose of waste per regulations

6.12 Equipment Cleaning
• Clean all equipment after use
• Follow equipment-specific procedures
• Remove all residue
• Document cleaning
• Prepare for next batch

7.	References

• NYS OCM Processing Regulations
• NFPA 30 Flammable Liquids Code
• OSHA Flammable Liquid Standards
• Source: https://cannabis.ny.gov/

8.	Definitions

Ethanol Extraction: A method using food-grade ethanol to dissolve cannabinoids and terpenes from cannabis plant material.

Cold Extraction: Extraction performed at very low temperatures to minimize chlorophyll and wax extraction.

Winterization: A process of removing waxes and lipids by dissolving extract in ethanol, freezing, and filtering.

Residual Solvent: Solvent remaining in the final product after extraction and purification.

C1D1/C1D2: Hazardous location classifications for areas where flammable gases or vapors may be present.

Related SOPs and Cross-References

Category: EXTRACTION - NYS OCM

The following SOPs are related to this procedure:

PROC-031: Solvent Recovery - Solvent reclamation

PROC-009: Distillation - Post-extraction processing

GEN-038: Chemical Control - Solvent safety

Note: This SOP is part of a comprehensive cannabis compliance system. SOPs marked "(OTHER STATES)" contain recommendations from other state programs and are suggestions, not NYS OCM requirements.