Standard Operating Procedure: Hydrocarbon Extraction

1.	Purpose

This Standard Operating Procedure establishes guidelines for hydrocarbon (butane/propane) extraction of cannabis to produce concentrates in compliance with NYS Office of Cannabis Management (OCM) safety and processing requirements.

2.	Scope

This SOP applies to all hydrocarbon extraction operations at licensed cannabis processing facilities in New York State using butane, propane, or blends thereof.

3.	Prerequisites

• Specialized training on hydrocarbon extraction
• Certification on closed-loop systems
• Understanding of explosion hazards
• Knowledge of hydrocarbon properties
• Emergency response training

4.	Responsibilities

Extraction Manager:
• Oversee all hydrocarbon operations
• Ensure regulatory compliance
• Approve extraction methods
• Maintain safety programs

Extraction Operators:
• Operate closed-loop systems
• Monitor all parameters
• Maintain equipment
• Follow safety protocols strictly

Safety Officer:
• Verify C1D1 compliance
• Monitor LEL detectors
• Conduct safety audits
• Lead emergency response

Quality Assurance:
• Verify residual solvent compliance
• Test product quality
• Review documentation
• Approve product release

5.	Materials

• Closed-loop extraction system
• N-butane and/or propane (instrument grade)
• Extraction columns
• Collection vessels
• Vacuum oven
• LEL detectors
• Explosion-proof equipment
• Fire suppression system
• PPE (non-static)
• Ground straps

6.	Procedure

6.1 Critical Safety Requirements
• MANDATORY C1D1 classified space
• All equipment explosion-proof
• Continuous LEL monitoring (<10% LEL)
• Emergency ventilation system
• Fire suppression (CO2 or clean agent)
• No ignition sources
• Static dissipating flooring
• Grounding and bonding required
• Emergency shutoffs accessible
• Written emergency procedures posted

6.2 Pre-Operation Safety Check
• Verify C1D1 room compliance
• Test LEL detectors (calibrate weekly)
• Check fire suppression status
• Verify ventilation operational
• Confirm emergency exits clear
• Inspect all electrical for compliance
• Document safety checklist

6.3 Equipment Inspection
• Inspect closed-loop system:
  - All connections tight
  - No visible leaks
  - Pressure relief valves functional
  - Sight glasses clear
  - Hoses in good condition
• Perform leak test with nitrogen
• Document inspection results
• Do not operate if any deficiency

6.4 Material Preparation
• Weigh input material accurately
• Pack material column:
  - Even distribution
  - Appropriate density
  - Note weight and strain
• Seal column properly
• Verify no air in system
• Document in tracking system

6.5 Solvent Loading
• Use only certified hydrocarbon
• Verify solvent quantity
• Load into recovery tank
• Record lot number and amount
• Ground all transfer containers
• Never open flame or spark

6.6 Extraction Process
• Cool extraction column (dry ice or chiller)
• Release solvent into material column
• Allow solvent to soak (per recipe)
• Collect extract in collection vessel
• Monitor temperature and pressure
• Recovery solvent to tank
• Multiple passes if needed

6.7 Closed-Loop Operation
• NEVER open system during operation
• Monitor pressures continuously
• Watch for pressure anomalies
• Use only approved recovery methods
• Maintain temperatures per recipe
• Document all parameters

6.8 Solvent Recovery
• Recover solvent back to tank
• Achieve maximum recovery
• Monitor recovery completion
• Do not heat above solvent limits
• Final vacuum purge on extract

6.9 Purging Residual Solvent
• Transfer extract to vacuum oven
• Purge at appropriate temperature:
  - Maintain below terpene degradation
  - Typical: 90-110°F
  - Full vacuum
• Duration: Until residual testing passes
• Flip/spread extract for complete purge
• Document purge parameters

6.10 Residual Solvent Testing
• Test every batch for residual solvents
• Maximum limits per OCM:
  - Butane: <5000 ppm
  - Propane: <5000 ppm
• Do not release if exceeds limits
• Continue purging if necessary
• Document test results

6.11 Product Documentation
• Complete batch record:
  - Input material and weight
  - Solvent used (lot, amount)
  - Extraction parameters
  - Purge conditions
  - Output weight
  - Yield calculation
  - Testing results
• Update tracking system

6.12 Post-Operation
• Ensure system fully recovered
• Safe shutdown procedures
• Ventilate area before opening
• Clean equipment
• Store solvent properly
• Document completion

7.	References

• NYS OCM Processing Regulations
• NFPA 58 Liquefied Petroleum Gas Code
• NFPA 30 Flammable Liquids Code
• OSHA 29 CFR 1910.106
• Source: https://cannabis.ny.gov/

8.	Definitions

Closed-Loop System: An extraction system where solvent is contained and recovered, never released to atmosphere.

C1D1: Class 1, Division 1 hazardous location where flammable gases may exist under normal conditions.

LEL (Lower Explosive Limit): The minimum concentration of a gas that will ignite; must stay below 10% of LEL.

Hydrocarbon: Compounds of hydrogen and carbon; butane and propane are common extraction solvents.

Purging: The process of removing residual solvent from extracted product using heat and vacuum.

Related SOPs and Cross-References

Category: EXTRACTION - NYS OCM

The following SOPs are related to this procedure:

PROC-031: Solvent Recovery - Solvent reclamation

GEN-038: Chemical Control - Hydrocarbon safety

GEN-009: Workplace Safety - Safety protocols

Note: This SOP is part of a comprehensive cannabis compliance system. SOPs marked "(OTHER STATES)" contain recommendations from other state programs and are suggestions, not NYS OCM requirements.