Standard Operating Procedure: Rosin Press Extraction

1.	Purpose

This Standard Operating Procedure establishes guidelines for solventless rosin extraction using heat and pressure to produce cannabis concentrates in compliance with NYS Office of Cannabis Management (OCM) requirements.

2.	Scope

This SOP applies to all rosin press operations at licensed cannabis processing facilities in New York State.

3.	Prerequisites

• Training on rosin press operation
• Understanding of heat/pressure parameters
• Knowledge of material preparation
• Familiarity with safety procedures
• Understanding of quality standards

4.	Responsibilities

Processing Manager:
• Oversee rosin operations
• Set processing parameters
• Approve production methods
• Monitor quality outcomes

Press Operators:
• Operate rosin press equipment
• Prepare input material
• Monitor pressing process
• Document production

Quality Assurance:
• Verify input material quality
• Inspect finished product
• Test for contaminants
• Approve batch release

5.	Materials

• Rosin press
• Rosin bags/mesh (various microns)
• Parchment paper (unbleached)
• Pre-press mold
• Temperature gun/probe
• Collection tools
• Scales
• Timer
• Gloves (heat resistant)
• Storage containers

6.	Procedure

6.1 Material Selection and Preparation
• Select appropriate starting material:
  - Flower (fresh or cured)
  - Hash/bubble hash
  - Dry sift/kief
• Assess material quality:
  - Moisture content (optimal 55-62%)
  - Freshness
  - Trichome quality
• Weigh and document input material

6.2 Pre-Press Preparation
• For flower:
  - Weigh appropriate amount (3-7g typical)
  - Pack into pre-press mold
  - Create uniform puck
• For hash/sift:
  - Select appropriate bag size (25-90μ)
  - Fill bag loosely
  - Seal bag properly

6.3 Rosin Press Setup
• Turn on press and heat plates
• Set temperature based on material:
  - Flower: 180-220°F
  - Hash: 160-190°F
  - Fresh press: Lower temperatures
• Allow plates to reach stable temperature
• Verify temperature with probe
• Prepare parchment paper

6.4 Pressing Procedure
• Place material between parchment:
  - Fold parchment to contain material
  - Position on lower plate
• Close press slowly
• Apply gradual pressure
• Hold for designated time:
  - Flower: 60-180 seconds
  - Hash: 45-90 seconds
• Monitor for rosin flow
• Release pressure slowly

6.5 Pressure and Time Guidelines
• Start with light pressure
• Increase gradually over 30 seconds
• Maximum pressure varies by material
• Watch rosin flow:
  - Too fast = too hot
  - No flow = too cold or dry
• Adjust parameters as needed
• Document optimal settings

6.6 Collection
• Remove parchment from press
• Allow to cool slightly
• Collect rosin from parchment:
  - Use clean collection tool
  - Avoid contamination
  - Collect all material
• Weigh collected rosin
• Calculate yield

6.7 Quality Assessment
• Evaluate rosin quality:
  - Color (lighter = fresher/better)
  - Consistency
  - Aroma
  - Contaminants
• Document quality observations
• Segregate different grades
• Note pressing parameters for future

6.8 Post-Processing (Optional)
• Rosin may be further processed:
  - Cold curing
  - Whipping
  - Jarring for budder/badder
• Document any post-processing
• Maintain proper storage

6.9 Yield Documentation
• Calculate and record yields:
  - Input weight
  - Output weight
  - Yield percentage
• Compare to expected yields:
  - Flower: 15-25%
  - Hash: 50-80%
• Investigate low yields
• Track batch-to-batch consistency

6.10 Equipment Cleaning
• Clean press plates after use
• Remove residue carefully
• Do not scratch plates
• Clean collection tools
• Organize work area
• Document cleaning

6.11 Batch Documentation
• Complete batch record:
  - Date and time
  - Input material (strain, weight)
  - Temperature
  - Pressure
  - Time
  - Yield
  - Quality notes
• Update tracking system
• Submit for testing

7.	References

• NYS OCM Processing Regulations
• Solventless Extraction Best Practices
• Source: https://cannabis.ny.gov/

8.	Definitions

Rosin: A solventless cannabis concentrate produced by applying heat and pressure to extract oils from plant material.

Micron: A unit of measurement (1/1000 of a millimeter) used to describe rosin bag mesh size.

Pre-Press: The process of compressing flower into a puck shape before pressing.

Yield: The percentage of output weight compared to input weight.

Cold Cure: A post-processing technique where rosin is stored at cool temperatures to achieve desired consistency.

Related SOPs and Cross-References

Category: EXTRACTION - NYS OCM

The following SOPs are related to this procedure:

PROC-023: Hash Production - Input material

PROC-021: Concentrate Packaging - Product packaging

GEN-011: Equipment Calibration - Press calibration

Note: This SOP is part of a comprehensive cannabis compliance system. SOPs marked "(OTHER STATES)" contain recommendations from other state programs and are suggestions, not NYS OCM requirements.