Standard Operating Procedure: Distillation

1.	Purpose

This Standard Operating Procedure establishes guidelines for short-path and wiped-film distillation of cannabis extracts to produce purified distillate in compliance with NYS Office of Cannabis Management (OCM) requirements.

2.	Scope

This SOP applies to all cannabis distillation operations at licensed processing facilities in New York State.

3.	Prerequisites

• Training on distillation equipment
• Understanding of vacuum systems
• Knowledge of boiling point principles
• Familiarity with cannabinoid properties
• Safety training for high-temperature operations

4.	Responsibilities

Processing Manager:
• Oversee distillation operations
• Set processing parameters
• Approve methods and equipment
• Ensure compliance

Distillation Operators:
• Operate distillation equipment
• Monitor process parameters
• Collect fractions properly
• Maintain equipment

Quality Assurance:
• Test distillate quality
• Verify potency
• Monitor for contaminants
• Approve batch release

5.	Materials

• Short-path distillation apparatus OR
• Wiped-film/thin-film distillation system
• Vacuum pump
• Heating mantle/system
• Chiller system
• Boiling flasks
• Collection flasks
• Vacuum gauge
• Temperature controllers
• PPE (heat resistant gloves, safety glasses)

6.	Procedure

6.1 Pre-Distillation Preparation
• Input material requirements:
  - Winterized and filtered crude
  - Decarboxylated (if not done)
  - Free of residual solvent
• Verify input quality
• Calculate expected yield
• Document input weight and source

6.2 Decarboxylation (if needed)
• Heat crude extract:
  - Temperature: 220-250°F
  - Time: 30-60 minutes
  - Under vacuum or nitrogen
• Monitor for bubble cessation
• Indicates conversion complete
• Document decarb parameters

6.3 Equipment Setup - Short Path
• Assemble distillation apparatus:
  - Boiling flask
  - Short path head
  - Condenser with chiller
  - Collection flasks (fractions)
  - Vacuum system
• Verify all connections sealed
• Test vacuum hold
• Set up heating mantle

6.4 Equipment Setup - Wiped Film
• Verify system clean
• Load feed vessel
• Set roller/wiper speed
• Configure heating zones
• Connect vacuum system
• Set condenser temperature
• Verify collection vessels

6.5 Vacuum and Temperature Settings
• Achieve deep vacuum:
  - Short path: 50-500 micron
  - Wiped film: Per manufacturer
• Set temperatures:
  - Terpene fraction: 120-150°C
  - Main body (cannabinoids): 160-200°C
  - Residue: Higher temperatures
• Temperature varies with vacuum depth

6.6 First Pass Distillation
• Apply heat gradually
• Monitor pressure and temperature
• Collect fractions:
  - Heads (terpenes, volatiles)
  - Main body (cannabinoids)
  - Tails (residual)
• Document collection times/temps
• Note color and clarity

6.7 Fraction Collection
• Heads fraction:
  - First material over
  - Contains terpenes
  - May contain residual solvent
  - Set aside or discard
• Main body:
  - Target cannabinoids
  - Clear to golden color
  - Majority of yield
• Tails fraction:
  - Darker color
  - Higher boiling compounds
  - May reprocess

6.8 Second Pass (if needed)
• For higher purity:
  - Re-distill main body
  - Same process
  - Tighter fraction cuts
• Achieves higher potency
• Document second pass

6.9 Quality Parameters
• Target specifications:
  - Potency: 80-95%+ cannabinoids
  - Color: Clear to light gold
  - Clarity: No cloudiness
  - Residual solvents: Pass limits
• Test each batch
• Document results

6.10 Post-Distillation Processing
• Transfer to storage containers
• Protect from light and heat
• May further process:
  - Add terpenes
  - Formulate products
  - Create cartridges
• Document all handling

6.11 Yield Documentation
• Record all weights:
  - Input crude weight
  - Heads fraction
  - Main body weight
  - Tails fraction
  - Total recovery
• Calculate yield percentage
• Compare to expected
• Document in batch record

6.12 Equipment Cleaning
• Allow system to cool
• Disassemble components
• Clean all glassware/components
• Remove residue completely
• Inspect for damage
• Document cleaning

7.	References

• NYS OCM Processing Regulations
• Distillation Equipment Manufacturer Guidelines
• Source: https://cannabis.ny.gov/

8.	Definitions

Distillation: A separation process using differences in boiling points to purify substances.

Short-Path Distillation: A distillation technique with minimal distance between evaporation and condensation surfaces.

Wiped-Film Distillation: A continuous distillation method using a thin film of material spread on a heated surface.

Fraction: A portion of distillate collected at specific temperature/time intervals.

Decarboxylation: The process of removing a carboxyl group from cannabinoids through heat, activating them.

Related SOPs and Cross-References

Category: REFINEMENT - NYS OCM

The following SOPs are related to this procedure:

PROC-005: Ethanol Extraction - Pre-distillation

PROC-040: Crystallization - Isolate production

PROC-020: Winterization - Pre-distillation processing

Note: This SOP is part of a comprehensive cannabis compliance system. SOPs marked "(OTHER STATES)" contain recommendations from other state programs and are suggestions, not NYS OCM requirements.