Standard Operating Procedure: Tincture Manufacturing

1.	Purpose

This Standard Operating Procedure establishes guidelines for manufacturing cannabis tinctures in compliance with NYS Office of Cannabis Management (OCM) regulations.

2.	Scope

This SOP applies to all cannabis tincture manufacturing operations at licensed facilities in New York State.

3.	Prerequisites

• Training on tincture formulation
• Understanding of extraction and infusion
• Knowledge of dosing calculations
• Familiarity with carrier oils/alcohols
• Quality control training

4.	Responsibilities

Production Manager:
• Oversee tincture manufacturing
• Approve formulations
• Ensure compliance
• Monitor quality

Production Staff:
• Prepare formulations
• Execute manufacturing
• Package products
• Document production

Quality Assurance:
• Test formulations
• Verify potency and dosing
• Approve batch release
• Monitor for contaminants

5.	Materials

• Cannabis extract or distillate
• Carrier medium (MCT oil, alcohol, etc.)
• Flavoring agents (if applicable)
• Mixing vessels
• Heating equipment
• Precision scales
• Homogenizer or mixer
• Dropper bottles
• Filling equipment
• Labels and packaging

6.	Procedure

6.1 Formulation Design
• Define tincture specifications:
  - Target potency (mg/mL)
  - Serving size (dropper)
  - Total bottle volume
  - Carrier type
• Calculate cannabinoid amounts:
  - Total mg per bottle
  - Extract amount needed
  - Carrier volume
• Document formulation

6.2 Carrier Selection
• Select appropriate carrier:
  MCT OIL:
  - Good absorption
  - Neutral flavor
  - Long shelf life
  
  ALCOHOL (if permitted):
  - Rapid sublingual absorption
  - Preservative properties
  - 60-80% ethanol typical
  
  VEGETABLE GLYCERIN:
  - Sweet taste
  - Slower absorption
• Verify carrier quality and source

6.3 Extract Preparation
• Verify extract quality:
  - Tested and approved
  - Known potency
  - Appropriate for tinctures
• Weigh extract precisely
• Document lot and weight
• Warm if needed for viscosity

6.4 Blending Process
• Combine extract with carrier:
  - Warm carrier slightly (120-140°F)
  - Add extract
  - Mix thoroughly
• Ensure homogeneity:
  - Use homogenizer for large batches
  - Mix until uniform
  - No visible separation
• Add flavoring if applicable

6.5 Potency Verification
• Sample for testing:
  - Multiple points in batch
  - Submit for potency analysis
• Verify target potency achieved
• Calculate actual mg/mL
• Document results

6.6 Filling Operations
• Set up filling equipment:
  - Calibrate volume
  - Verify accuracy
  - Clean and sanitize
• Fill bottles:
  - Consistent fill volume
  - Minimize air incorporation
  - Avoid spillage
• Attach droppers/closures

6.7 Fill Accuracy
• Verify fill volume:
  - Weigh samples
  - Calculate variance
  - Target: ±5% of specified volume
• Document fill weights
• Segregate out-of-spec units

6.8 Labeling Requirements
• Labels must include:
  - Product name
  - Total cannabinoid content
  - Serving size and servings per container
  - Suggested use instructions
  - Ingredient list
  - Required warnings
  - Batch/lot number
• Verify label accuracy
• Apply labels properly

6.9 Packaging
• Package per OCM requirements:
  - Child-resistant closure
  - Measured dropper
  - Required warnings
• Include dosing guide if applicable
• Secondary packaging as needed

6.10 Quality Control
• Test finished product:
  - Potency verification
  - Homogeneity testing
  - Microbial testing
  - Visual inspection
• Hold until results received
• Document all testing

6.11 Stability Considerations
• Protect from:
  - Light exposure
  - Heat
  - Oxidation
• Consider shelf life:
  - Typical: 12-24 months
  - Establish expiration dates
• Document storage requirements

6.12 Batch Documentation
• Complete batch record:
  - Formulation details
  - Materials used (lots)
  - Process parameters
  - Fill data
  - Test results
  - Yield
• Update tracking system

7.	References

• NYS OCM Processing Regulations
• NYS OCM Packaging Requirements
• Source: https://cannabis.ny.gov/

8.	Definitions

Tincture: A liquid cannabis product typically administered sublingually using a dropper.

MCT Oil: Medium-chain triglyceride oil, commonly derived from coconut, used as a carrier.

Sublingual: Administration under the tongue for absorption through oral mucosa.

Homogeneity: Uniform distribution of cannabinoids throughout the product.

Potency: The concentration of cannabinoids, typically expressed as mg/mL for tinctures.

Related SOPs and Cross-References

Category: INFUSION - NYS OCM

The following SOPs are related to this procedure:

PROC-005: Ethanol Extraction - Extract production

PROC-026: Infused Product Formulation - Formulation

GEN-019: Packaging Labeling - Tincture labeling

Note: This SOP is part of a comprehensive cannabis compliance system. SOPs marked "(OTHER STATES)" contain recommendations from other state programs and are suggestions, not NYS OCM requirements.