Standard Operating Procedure: Decarboxylation

1.	Purpose

This Standard Operating Procedure establishes guidelines for decarboxylating cannabis material to convert acidic cannabinoids to their active forms in compliance with NYS Office of Cannabis Management (OCM) requirements.

2.	Scope

This SOP applies to all decarboxylation operations at licensed cannabis processing facilities in New York State.

3.	Prerequisites

• Understanding of decarboxylation chemistry
• Training on heating equipment
• Knowledge of temperature control
• Familiarity with cannabinoid conversion
• Safety training for heat operations

4.	Responsibilities

Processing Manager:
• Oversee decarboxylation operations
• Approve parameters
• Ensure quality
• Monitor compliance

Processing Technicians:
• Execute decarboxylation
• Monitor temperatures
• Document processes
• Maintain equipment

Quality Assurance:
• Verify conversion
• Test potency
• Review documentation
• Approve procedures

5.	Materials

• Decarboxylation oven or reactor
• Temperature controller
• Thermometers (calibrated)
• Vacuum system (if applicable)
• Timer
• Heat-resistant containers
• Personal protective equipment
• Documentation forms

6.	Procedure

6.1 Understanding Decarboxylation
• Chemical conversion process:
  - THCA → THC + CO2
  - CBDA → CBD + CO2
  - Removes carboxyl group
• Required for:
  - Edibles
  - Tinctures (oral)
  - Some concentrates
• Not required for:
  - Smokable products
  - Already decarbed materials

6.2 Material Preparation
• Verify material suitable:
  - Cannabis flower
  - Trim
  - Crude extract
  - Distillate (often pre-decarbed)
• Weigh material
• Record in batch documentation
• Spread evenly in containers

6.3 Equipment Setup
• Oven/reactor preparation:
  - Clean and dry
  - Verify calibration
  - Set temperature controller
• Temperature settings:
  - Flower: 220-250°F (105-121°C)
  - Extract: 200-240°F (93-115°C)
• Preheat to target temperature
• Verify temperature accuracy

6.4 Decarboxylation Process
• Place material in oven:
  - Spread in thin layer
  - Ensure heat access
  - Close securely
• Process duration:
  - Flower: 30-60 minutes
  - Extract: 20-40 minutes
• Monitor throughout process
• Do not exceed temperatures

6.5 Monitoring Completion
• Indicators of completion:
  - Minimal bubbling (extracts)
  - Color change (slight darkening)
  - Time/temperature achieved
• Do not over-decarboxylate:
  - Degrades cannabinoids
  - Creates CBN
  - Reduces potency
• Document completion time

6.6 Vacuum-Assisted Decarboxylation
• For extracts (optional):
  - Apply vacuum
  - Reduces boiling point
  - Preserves terpenes
  - Monitor pressure and temp
• Observe bubble cessation
• Release vacuum slowly

6.7 Cooling
• After decarboxylation:
  - Remove from heat
  - Allow controlled cooling
  - Protect from light
• Cool to room temperature
• Do not seal until cool (moisture)
• Document cooling

6.8 Quality Verification
• Test for conversion:
  - THCA/THC ratio
  - CBDA/CBD ratio
• Target: >85% conversion typical
• Reconvert if insufficient
• Document test results

6.9 Post-Decarboxylation Handling
• Transfer to storage:
  - Airtight containers
  - Cool, dark storage
  - Label with decarb status
• Proceed to formulation
• Document transfer

6.10 Batch Documentation
• Complete batch record:
  - Material ID and weight
  - Equipment used
  - Temperature and time
  - Observations
  - Completion indicators
  - Testing results
• Update tracking system

6.11 Equipment Cleaning
• Clean after use:
  - Allow to cool
  - Remove residue
  - Clean containers
• Document cleaning
• Prepare for next batch

6.12 Safety Considerations
• Heat hazards:
  - Use heat-resistant gloves
  - Avoid direct contact
  - Allow cooling before handling
• Ventilation:
  - CO2 release during process
  - Adequate ventilation required
  - Odor management

7.	References

• NYS OCM Processing Regulations
• Cannabis Chemistry Principles
• Source: https://cannabis.ny.gov/

8.	Definitions

Decarboxylation: The chemical reaction that removes a carboxyl group from cannabinoids through heat, converting them to active forms.

THCA (Tetrahydrocannabinolic Acid): The acidic, non-psychoactive precursor to THC found in raw cannabis.

CBDA (Cannabidiolic Acid): The acidic precursor to CBD found in raw cannabis.

CBN (Cannabinol): A cannabinoid formed through degradation or oxidation of THC.

Conversion Rate: The percentage of acidic cannabinoids converted to their neutral forms.

Related SOPs and Cross-References

Category: PROCESSING - NYS OCM

The following SOPs are related to this procedure:

PROC-010: Edibles Manufacturing - Edible production

PROC-026: Infused Product Formulation - Activation

GEN-011: Equipment Calibration - Oven calibration

Note: This SOP is part of a comprehensive cannabis compliance system. SOPs marked "(OTHER STATES)" contain recommendations from other state programs and are suggestions, not NYS OCM requirements.