Standard Operating Procedure: Winterization

1.	Purpose

This Standard Operating Procedure establishes guidelines for winterizing cannabis extracts to remove waxes, lipids, and fats for cleaner, purer products in compliance with NYS Office of Cannabis Management (OCM) requirements.

2.	Scope

This SOP applies to all winterization operations at licensed cannabis processing facilities in New York State.

3.	Prerequisites

• Training on winterization techniques
• Understanding of solvent handling
• Knowledge of filtration methods
• Familiarity with freezer operations
• Safety training for cold operations

4.	Responsibilities

Processing Manager:
• Oversee winterization operations
• Approve procedures
• Ensure safety compliance
• Monitor quality

Processing Technicians:
• Execute winterization
• Monitor temperatures
• Perform filtrations
• Document processes

Quality Assurance:
• Verify product quality
• Test for improvements
• Review documentation

5.	Materials

• Cannabis crude extract
• Food-grade ethanol
• Freezer (-20°C or colder)
• Mixing vessels
• Filter apparatus
• Filter media (various microns)
• Buchner funnel
• Vacuum pump
• Rotary evaporator
• Cold-resistant gloves
• Safety glasses

6.	Procedure

6.1 Purpose of Winterization
• Removes undesirable compounds:
  - Plant waxes
  - Lipids/fats
  - Chlorophyll (partial)
• Benefits:
  - Cleaner product
  - Better for vaping
  - Improved clarity
• Typically follows warm extraction

6.2 Material Assessment
• Evaluate crude extract:
  - Source and extraction method
  - Winterization needed?
  - Current clarity/consistency
• Cold-extracted material may not require
• Weigh and document starting material

6.3 Ethanol Addition
• Calculate ethanol needed:
  - Ratio: 10:1 to 20:1 (ethanol:extract)
  - More ethanol = better separation
• Warm extract slightly for mixing
• Add ethanol gradually
• Mix until fully dissolved
• Document amounts used

6.4 Cold Precipitation
• Transfer to freezer-safe vessel
• Place in freezer:
  - Temperature: -20°C minimum
  - Colder is better (-40°C ideal)
  - Duration: 24-48 hours
• Waxes/lipids precipitate out
• Solution becomes cloudy
• Do not disturb during precipitation

6.5 Filtration Setup
• Pre-chill filtration equipment
• Set up filter system:
  - Buchner funnel or filter vessel
  - Filter paper (various microns)
  - Vacuum source
• Keep cold during filtration
• Work quickly

6.6 Filtration Process
• Remove solution from freezer
• Filter immediately:
  - Pour through filter
  - Apply vacuum
  - Do not let warm up
• Multiple passes may be needed:
  - Coarse filter first
  - Fine filter second
• Collect filtered solution

6.7 Filter Media Selection
• Typical progression:
  - 25 micron initial
  - 5 micron secondary
  - 1 micron polish (optional)
• May use:
  - Paper filters
  - Celite/diatomaceous earth
  - Activated carbon (color)
• Document filter types used

6.8 Solvent Recovery
• Transfer filtrate to evaporator
• Remove ethanol:
  - Rotary evaporator
  - Falling film evaporator
  - Low temperature (below 120°F)
• Recover solvent for reuse
• Monitor residual solvent

6.9 Product Assessment
• Evaluate winterized extract:
  - Improved clarity
  - Lighter color
  - Different texture
• Compare to starting material
• Document improvements

6.10 Yield Calculation
• Weigh final product
• Calculate yield:
  - Input weight
  - Output weight
  - Percentage recovery
• Typical loss: 10-20%
• Document yield

6.11 Quality Testing
• Test winterized extract:
  - Potency
  - Residual solvent (<5000 ppm ethanol)
  - Appearance/clarity
• Compare to pre-winterization
• Document results

6.12 Documentation
• Complete batch record:
  - Starting material
  - Ethanol used
  - Temperatures and times
  - Filtration details
  - Final yield
  - Testing results
• Update tracking system

7.	References

• NYS OCM Processing Regulations
• Extraction Best Practices
• Source: https://cannabis.ny.gov/

8.	Definitions

Winterization: A process using cold temperatures and ethanol to precipitate and remove waxes, lipids, and fats from cannabis extract.

Precipitation: The formation of solid particles from a solution due to temperature or chemical changes.

Crude Extract: Unrefined cannabis extract containing cannabinoids plus waxes, lipids, and plant matter.

Lipids: Fat-soluble compounds including waxes and plant fats extracted from cannabis.

Filtrate: The liquid that passes through a filter, containing the desired compounds.

Related SOPs and Cross-References

Category: REFINEMENT - NYS OCM

The following SOPs are related to this procedure:

PROC-005: Ethanol Extraction - Extraction

PROC-009: Distillation - Post-winterization

GEN-016: Cold Storage - Cold processing

Note: This SOP is part of a comprehensive cannabis compliance system. SOPs marked "(OTHER STATES)" contain recommendations from other state programs and are suggestions, not NYS OCM requirements.