Standard Operating Procedure: Powder/Capsule Blend

1.	Purpose

This Standard Operating Procedure establishes guidelines for blending cannabis powders for capsule and other applications in compliance with NYS OCM requirements.

2.	Scope

This SOP applies to all powder blending operations at licensed cannabis processing facilities in New York State.

3.	Prerequisites

• Training on powder blending
• Understanding of uniformity requirements
• Knowledge of excipients
• Familiarity with GMP requirements
• Quality control training

4.	Responsibilities

Production Manager:
• Oversee blending operations
• Approve formulations
• Ensure GMP compliance
• Monitor quality

Production Staff:
• Execute blending procedures
• Monitor blend quality
• Document activities
• Maintain equipment

Quality Assurance:
• Test blend uniformity
• Verify potency
• Approve releases

5.	Materials

• Cannabis powder/extract
• Excipients/fillers
• Blending equipment
• Screens/sieves
• Scales
• Sample containers
• PPE

6.	Procedure

6.1 Formulation
• Design blend formula:
  - Active ingredient amount
  - Excipient selection
  - Total batch size
  - Particle size requirements
• Document formula
• Calculate amounts

6.2 Ingredient Preparation
• Prepare ingredients:
  - Weigh accurately
  - Sieve if needed
  - Remove agglomerates
  - Document lots
• Verify quality
• Stage for blending

6.3 Equipment Preparation
• Prepare blender:
  - Clean and dry
  - Appropriate size
  - Proper type
  - Verified clean
• Document preparation

6.4 Geometric Dilution
• If significant potency difference:
  - Add active to equal portion excipient
  - Blend thoroughly
  - Add equal portion, blend
  - Continue to total amount
• Ensures uniformity
• Document process

6.5 Blending Process
• Blend per specifications:
  - Load order
  - Blend speed
  - Blend time
  - Temperature monitoring
• Follow SOP exactly
• Document parameters

6.6 In-Process Checks
• During blending:
  - Visual uniformity
  - Sample for testing
  - Note any issues
  - Adjust if needed
• Document checks

6.7 Blend Uniformity Testing
• Test uniformity:
  - Sample from multiple locations
  - Test each sample
  - Calculate RSD
  - Target: <5% RSD
• Document results
• Reblend if needed

6.8 Potency Verification
• Verify target potency:
  - Test composite sample
  - Compare to target
  - Calculate accuracy
  - Within ±10% of target
• Document results

6.9 Particle Size
• Verify particle size:
  - Sieve analysis
  - Appropriate for application
  - No agglomerates
  - Flowability acceptable
• Document results

6.10 Discharge and Storage
• Discharge blend:
  - Into clean containers
  - Properly labeled
  - Sealed appropriately
  - Documented
• Store properly
• Track inventory

6.11 Equipment Cleaning
• Clean equipment:
  - Between products
  - Remove all residue
  - Document cleaning
  - Verify clean
• Prevent cross-contamination

6.12 Documentation
• Maintain records:
  - Batch record
  - Blend parameters
  - Test results
  - Yield data
• Update tracking
• Retain records

7.	References

• NYS OCM Processing Regulations
• Pharmaceutical Blending Guidelines
• Source: https://cannabis.ny.gov/

8.	Definitions

Blending: The process of combining powders to achieve uniform distribution.

Geometric Dilution: A mixing technique for achieving uniformity when combining small amounts with larger amounts.

RSD (Relative Standard Deviation): A measure of variability used to assess blend uniformity.

Excipient: An inactive ingredient used as a carrier or filler.

Related SOPs and Cross-References

Category: PROCESSING - NYS OCM

The following SOPs are related to this procedure:

PROC-024: Capsule Manufacturing - Capsule production

GEN-031: GMP Program - GMP compliance

PROC-019: Decarboxylation - Activation

Note: This SOP is part of a comprehensive cannabis compliance system. SOPs marked "(OTHER STATES)" contain recommendations from other state programs and are suggestions, not NYS OCM requirements.